Guide Specification for

Sky Factory SkyView™

Digital Cinema Virtual Skylight

SECTION 27 41 14

Video SKYLIGHT

Sky Factory's SkyView™ is a digital cinema virtual skylight that brings the dynamic beauty of nature – image and motion  – to interior environments. A high-tech illusion, SkyView features a ceiling-mounted industrial-grade LCD monitor in a trimmed skylight. SkyView utilizes RED ONE Digital Cinema Camera technology to capture proprietary high definition wild nature footage.

Specification Coordination: Edit this guide specification to meet project requirements. Coordinate with other specification sections as required. This guide specification is available in word processing format at www.SkyFactory.com. Display hidden text to see or print notes to specifier.

Drawing and Schedule Coordination: Show locations and sizes of video skylights, special perimeter details, and other conditions if required. Show electrical and communications circuits. Show locations of controls. Ensure that partitions to receive SkyView have required ceiling cavity depth and that integrity of fire rated and sound rated enclosures is maintained. Maintain access and ventilation clearances recommended by Sky Factory. Locate monitors and furniture to limit likelihood of accidental contact with monitors.

Other Products from Sky Factory: Contact Sky Factory for specifications.

Luminous SkyCeilings™ luminous sky-image ceilings.

Luminous Virtual Window™ edge-illuminated photo murals.

eScape™ 2.0 Digital Cinema (high-definition video) presentation of landscapes and seascapes.

eSea™ Digital Cinema (high-definition video) presentation of underwater environments.

Additional Information and Sky Design Assistance: Contact Sky Factory at:

Telephone: 641-472-1747

E-mail: info@SkyFactory.com

Internet:www.SkyFactory.com

- GENERAL

      - SUMMARY

        - Section includes digital cinema virtual skylights, consisting of the following:

          - Video monitor.

          - Video content.

          - Playback device.

          - Video controls.

- 5.	Mounting structure with angle reveal and dust cover.

Usually retain subparagraph below. Coordinate with "Network Attachment" paragraph in Part 2.

- 6.[Network attachment.]

      - SUBMITTALS

        - Comply with Section [01 33 00 – Submittal Procedures.] [_________.]

Expand submittal type descriptions below if Division 01 does not explain requirements for action, informational, and closeout submittals.

        - Action Submittals:

          - Product Data: Submit for each type of product specified.

          - Shop Drawings: Show construction, size and shape of digital cinema virtual skylights; identification of video content; wiring diagram; and coordination with adjacent work.

          - Samples:

            - Video Content: Provide link to web site video catalog. www.skyfactory.com/products/SkyView/

        - Informational Submittals:

          - Installation Instructions: Submit manufacturer’s installation instructions.

        - Closeout Submittals:

          - Operation and Maintenance Data: Submit operation and maintenance instructions.

          - Warranty Documentation: Submit manufacturer’s limited two-year warranty.

      - QUALITY ASSURANCE

        - Comply with Section [01 43 00 – Quality Assurance.] [_________.]

      - DELIVERY, STORAGE AND HANDLING

        - Comply with Section [01 66 00 – Product Storage and Handling Requirements.] [_________.]

        - Deliver products in manufacturer’s original packaging. Store materials indoors in location that is secure, dry, and stable in temperature. Handle in accordance with manufacturer’s instructions to prevent damage.

      - AMBIENT CONDITIONS

        - Do not install until building is enclosed, permanent HVAC equipment is working and ambient temperature and humidity have stabilized within units' operating ranges.

      - WARRANTY

        - Comply with Section [01 78 36 – Warranties.] [_________.]

        - Provide manufacturer’s two-year limited video monitor warranty and one-year limited system warranty.

- PRODUCTS

      - DIGITAL CINEMA VIRTUAL SKYLIGHTS

        - Acceptable Manufacturer:

          - Provide SkyView™ digital cinema virtual skylights as manufactured by Sky Factory, Inc., phone 641-472-1747, fax 641-472-1014, www.SkyFactory.com, info@SkyFactory.com.

Sky Factory’s SkyView is a digital cinema virtual skylight that uses multisensory stimuli, image and motion to create a simulated portal to a natural exterior. Its unique design features include proprietary, horizontally-oriented nature scenes mastered to deliver real-time, unedited (long form) content of wild nature, displayed on professional-caliber monitors.

TELEVISED CONTENT IS NOT COMPARABLE. Our installations are designed as architectural features that create the illusion of perceived space for indoor wellness. We are unaware of another manufacturer that comes close to meeting our quality. Please discuss this with the Owner and consider specifying Sky Factory as the only acceptable manufacturer. We will be happy to provide you price quotations so you can make your own determination of value.

          - Substitutions: [Comply with provisions of [Section 01 25 00] [_________] for substitution procedures.] [Not allowed.]

        - General Requirements:

          - Provide a complete system of components from a single manufacturer.

Contact Sky Factory for UL equipment standards available, such as for health care environment requirements.

- Units shall operate with power characteristics shown on Drawings.

          - Units shall operate within the following ranges:

            - Power: 100-240V AC; 50-60 Hz												Model 4625—278 Watts; Model 5530—408 Watts.

            - Temperature: 41° to 95° F.

            - Humidity: 20 to 80 percent, non-condensing.

        - Ceiling mounting structure: Recessed case with integrated playback and venting, sized for monitor indicated.

          - Dust cover: .040 sheet aluminum with baked enamel, semi-gloss white color.

          - Angle reveal: Extruded aluminum with powder coat, semi-gloss white color.

          - Communications Wiring: Provide to junction box in dust cover at top.

          - Power: Provide to junction box in dust cover at top.

          - Fasteners: As supplied by manufacturer.

LED monitors contain no mercury and comply with Energy Star Version 4.0 for computer monitors.

Drawing Coordination: Provide 10 5/8-inch minimum deep ceiling openings for LED monitors.

        - Monitor: Commercial LED-lit LCD monitor.

Monitor dimensions are nominal; actual viewable area of video is slightly more than dimensions indicated. Contact Sky Factory for currently available monitors.

          - Size: [46x25] [55x30] inches viewable.

        - Video Playback Device:

          - Output Format: HDMI.

Select video content from the most current programs available at Sky Factory's web site:www.skyfactory.com/products/SkyView/. Select programs whose durations add up to your required program lengths. Eight, twelve or 16 hour packages of content. Edit the Programs subparagraphs according to your selections. When new video content is purchased, provided by simple plug and play module without the cost or disruption of an on-site maintenance visit.

          - Video Content: By Sky Factory.

            - Quality: 1920 by 1080 pixel high-resolution videos, filmed in real time.

            - Programs: [8][12][16][_____] hours minimum.

              - Spring Sky: Afternoon to Night with Blossoming Hawthorne, SV024, 189 minutes.

              - Four Firs and Mountain Clouds, SV025, 69 minutes.

              - Autumn Sunset and Nightfall, SV023, 60 minutes.

              - Summer Sky with Pines and Cliff, SV020, 78 minutes.

              - Winter Sky: Creation, Dissolution, Creation, SV019, 73 minutes.

              - Autumn Late Afternoon with Gingko and Pine, SV014, 94 minutes.

              - Summer Early Morning Sky, SV013, 102 minutes.

              - [_____]

              - Programs selected by [Architect][_____] from manufacturer's full range.

        - Controls: In-wall control system.

          - Powered by Cat6 from video player.

          - Touch-screen display, 1 3/4 inches wide by 2 1/4 inches tall.

          - Bezel, 4 5/8 inches wide by 4 3/8 inches tall.

          - Button bezel and faceplate color: As selected by [Architect] [______] from manufacturer's full range.

Junction boxes for pre-wiring in-wall control units are available for installation in new partitions. Retrofit fins are standard with in-wall control units.

          - [Junction Box: Low voltage mounting bracket provided.]

          - Mounting: [Manufacturer's standard screw attachment.]

      - ACCESSORIES

        - Provide fasteners, blocking, and other accessories required for complete installation.

        - Power Wiring: As specified in [Division 26 - Electrical] [________] and as recommended by video skylight manufacturer.

        - Communications Cabling: Cat6 [plenum rated,] [as specified in Division 27 – Communications and] as recommended by video skylight manufacturer.

- EXECUTION

      - EXAMINATION

        - Examine areas receiving work of this Section to identify conditions that may adversely affect installation. Do not begin installation until adverse conditions have been remedied.

Coordinate the following subparagraph with cable requirements in Part 2.

          - Verify that cable paths comply with video skylight manufacturer's recommendations.

      - PREPARATION

If opening is in sound rated or fire-rated ceiling, show details of opening on Drawings.

        - Openings to receive digital cinema virtual skylights shall be sized according to manufacturer’s requirements. Provide wiring access as indicated in manufacturer's installation details.

      - INSTALLATION

        - Install in accordance with manufacturer’s instructions and shop drawings.

        - Install mounting structure with monitor and playback equipment.

        - Make connections to power as specified in [Division 26 - Electrical] [________].

        - Make visual connections and control connections as specified in [Division 27 – Communications] [_____].

Delete paragraph below if network attachment is by Owner or is not required.

          - [Network Attachment: Connect router to each in-wall control unit with cabling specified.]

        - Install in-wall control unit.

      - CLEANING

        - Clean units according to manufacturer’s usage guide.

      - PROTECTION

        - Protect units from construction activities according to manufacturer’s instructions.

          - Disconnect power to devices.

          - Seal devices against dust and overspray.

          - Protect against impact. Provide signage, caution tape, or barriers as required.

Schedule can be used, if necessary, to indicate video content or other requirements for digital cinema virtual skylights. To reduce potential for coordination errors, do not repeat information shown on Drawings. Examples are given below.

      - SCHEDULE

| ROOM | SIZE | VIDEO | COMMENT |
| --- | --- | --- | --- |
| 112 Waiting Room | 46x25 inches | Programs SV020, SV014, SV024, SV010, SV013. | SkyLight is over central table |
| 135 Procedure Room | 55x30 inches | Playlist "Trees" SV024, SV025, SV020, SV014, SVo12, SV010 | Control with black bezel located in Nurse Station. |
| 143 Imaging Suite | 46x25inches | Programs SV013, SV023, SV007, SV019 |  |

END OF SECTION

SkyView™ and eScape™ are trademarks of Sky Factory, Inc. Copyright 2017 by Sky Factory, Inc.; qualified design and construction professionals may copy for preparation of construction specifications. Issued 11/4/20.